Calculating the Inductance of an Inductor (H)

Alexander Cristaudo

CRSALE010

Date Performed: 18 October 2022

Peoplesoft ID: 1873229

Lab Test 2

# Introduction and Aim

In an RL circuit, which is a circuit consisting of both a resistor and an inductor, the impedance of the circuit is equivalent to the total resistance of the circuit from both the inductor and resistor. Now the current in the circuit is related to both the resistance and impedance in the following way: I=VRR=VinZ where VR is the potential difference across the resistor and Vin is the input potential difference supplied to the whole circuit (by a frequency generator in this case). From this equation, we get that Vin=RZVR (Equation (1)). Now R and Z are fixed, so this gives a linear relationship between Vin and VR. There is also a relationship between the impedance, resistance, frequency (of the alternating current) and the inductance of an inductor. This relationship is described by the equation Z2=R2+4π2f2L2 (Equation (2)) where L is the inductance and f is the frequency supplied by the frequency generator. The aim of this experiment is to calculate the inductance of an inductor in an RL circuit and check the validity of this result.

# Method and Apparatus

## Apparatus

- An oscilloscope, to measure the frequency and Vin and VR

- A multimeter, to measure the resistance of a resistor

- An inductor

- A resistor

- A frequency generator

- Cables to connect the frequency generator to the inductor, inductor to the resistor , resistor to the frequency generator and to use the multimeter to calculate the resistance

- 2 Channel cables to get VR and Vin

Diagram 1

Frequency Generator

Resistor

Inductor

A diagram showing the apparatus required

## Method

The multimeter was first used to get the resistance of resistor number 49. This was obtained by connecting 2 leads to the multimeter, one into the COM port and the other into VΩ. The mode of the multimeter was changed to Ω (to turn the multimeter into an ohmmeter to measure the resistance), then the other sides of the leads were connected to the resistor, so as to connect the resistor in series with the multimeter. The resistance is displayed on the multimeter screen and the range button was pressed until the resistance was displayed to the most digits. After the resistance was measured, the circuit in Diagram 1 was constructed. Firstly one lead connected the frequency generator to the inductor, then another lead connected the inductor to the resistor, finally a lead connected the resistor to the other end of the frequency generator. Some frequency was selected on the frequency generated and not changed afterwards (by adjusting the frequency dial). The channel cables were then used such that Channel 1 calculated Vin and Channel 2 calculated VR. This was done by connecting the Channel 1 cables across the frequency generator and Channel 2 cables across the resistor. To get the potential difference displayed on the oscilloscope, the Vpeak to peak mode was turned on by clicking the Measure button, then Ch1 and using the multipurpose switch to turn on frequency and Peak-to-Peak. Then the frequency was measured along with Vpeak to peak for Channel 1 (Vin) and Vpeak to peak for Channel 2 (VR). After this was recorded, the amplitude dial on the frequency generator was moved to adjust Vin. Recordings for this Vin and VR were recorded and this was repeated until 6 readings were obtained.

# Data and Results

## Table

Table 1: The raw data for VR and Vin in a table

The potential difference across the resistor (VR) (V) for an input voltage Vin (V) for the RL circuit

| VR (V) | Vin (V) |
| --- | --- |
| 0.192 | 2.16 |
| 0.128 | 1.48 |
| 0.272 | 3.14 |
| 0.340 | 3.86 |
| 0.428 | 4.96 |
| 0.244 | 2.80 |

## Values

The resistance was measured to be 21.8 Ω

The period was measured to be 0.0003510s

## Graph

Now we plot the data in Table 1. This is done to work out the gradient and intercept to work out the impedance Z

Graph 1: The data in Table 1 plotted as a linear graph

The potential difference across the resistor (VR) (V) plotted against its input voltage Vin (V) for the RL circuit

The input potential difference Vin (V) supplied by the frequency generator

The potential difference across the resistor, VR(V) for an input potential difference Vin (V)

# Analysis

## Calculating the Uncertainty of the Resistance

This was a digital reading, so a rectangular PDF. Thus, uRreading=21.85-21.7523=0.0288

The multimeter was rated at 1%, so uRrating=21.8×1%=0.218

The full uncertainty budget for the resistance of the resistor:

| Uncertainty Component | Standard Uncertainty (Ω) | Type of Uncertainty |
| --- | --- | --- |
| uRreading | 0.0288 | Type B |
| uRrating | 0.218 | Type B |
| uCR=0.02882+0.2182=0.220 |  |  |

So R=21.800±0.220

## Calculating the Uncertainty of the Frequency

First the frequency is related to the period by f=1t⇒f=2849

Now we know that f=t-1⇒uf=ut

So we can use the frequency reading instead of the period to calculate the uncertainty (as they both are to the same number of significant figures)

This was a digital reading, so ufreading=2849.5-2848.523=0.289 Hz

The oscilloscope was rated at 2% so ufrating=2849×2%=57.0 Hz

The full uncertainty budget for the frequency:

| Uncertainty Component | Standard Uncertainty (Hz) | Type of Uncertainty |
| --- | --- | --- |
| ufreading | 0.289 | Type B |
| ufrating | 57.0 | Type B |
| uCf=0.2892+57.02=57.0 |  |  |

So f=2849.0±57.0 Hz

## Calculating the Impedance

From Graph 1, using the Linear Least Squares method, it was obtained that:

m=11.5911 ±0.1580

c=-0.0320 ±0.0450

where m and c is the gradient and intercept respectively.

Now from Equation 1, Vin=RZVR so for Y=Vin and X=VR, we get m=RZ

Thus the best approximation for Z=mR=11.5911 ×21.8 = 253

From m=RZ=RZ-1, we get that um=m1uRR2+-1uZZ2

So 0.1580=11.59110.22021.82+uZ2532⇒uZ=2.32

Thus Z=253.00±2.32 Ω

## Calculating the Inductance

From Equation (2), Z2=R2+4π2f2L2

So Z2-R2=4π2f2L2⟺L2=14π2Z2-R2f-2 (3)

Thus the best estimate for L=14π22532-21.822849-2=0.0141H

Now we have that for A=L2,uA=A2uLL2=2LuL. Thus uL2=2LuL

But by Equation (3), we have that uA=L2uZ2-R2Z2-R22+-2uff2

Now we have shown that uX2=2XuX for some variable X, so using this for Z and R:

uZ2=2ZuZ=2×253×2.32=1.17×103

uR2=2RuR=2×21.8×0.220=9.59

Thus uZ2-R2=uZ22+uR22=1.17×1032+9.592=1.17×103

Now we have that 2LuL=uA=L2uZ2-R2Z2-R22+-2uff2

So 2LuL=L2uZ2-R2Z2-R22+-2uff2⟺uL=12LuZ2-R2Z2-R22+-2uff2

So uL=120.01411.17×1032532-21.822+257.028492=0.000311

Thus we have that L=0.014100±0.000311 H

# Discussion

From Equation (1), Vin=RZVR so theoretically Graph 1 should have an intercept at 0. It was calculated that 
c=-0.0320 ±0.0450

Now this means -0.077 ≤c ≤0.013

Now 0 lies in this interval, so this is a valid result.

Factors influencing the measurements include:

- The multimeter and oscilloscope only display the resistance and potential difference up to 3 significant figures.

- The frequency on the oscilloscope is not constant and jumps around every second between some interval. This leads to the measurement of the frequency to be a best estimate as to what the oscilloscope is showing.

# Conclusion and Recommendations

## Conclusion

It was calculated that L=0.014100±0.000311 H and the aim has been met since the calculated results line up with the theoretical expectations for the equations (as mentioned in the Discussion).

## Sources of Uncertainty

- The uncertainty related with the reading off of the oscilloscope screen

- The uncertainty related to the rating of the oscilloscope

- The uncertainty related to the rating of the multimeter

- The uncertainty related to the reading off of the multimeter

- The frequency jumps on the oscilloscope (not constant over time)

## Recommendations

- Use a more accurate multimeter

- Have an oscilloscope which provides more significant figures. This will provide stronger results and have the frequency not jump as much as it does.

- Use the frequency stated on the frequency generator as this has more significant figures