Capacitance of a Capacitor

Peoplesoft ID: 1873229

Alexander Cristaudo

Student Number: CRSALE010

The RC Circuit

Date Performed: 20 September 2022

Partner: Michael Haydam

Introduction and Aim

An RC Circuit is a circuit consisting of a resistor, capacitor and, in this case, a frequency generator that drives the current through the circuit. In this arrangement, the resistor controls the charging and discharging of the capacitor. The frequency generator provides a potential difference Vin, with part of this applied across a resistor, denoted by VR. The impedance, equivalent to the total resistance for an RC circuit, is given by:

Z=R2+1ωC2

where C is the capacitance of a capacitor, and ω= 2 πf, the period of oscillation provided by the frequency generator. The peak current is also provided by I0=VRR=VinZ. Using this, the following equation forms: VinVR2=12πRC21f2+1. The aim is thus to use the frequency supplied by the frequency generator to calculate C and to calculate C theoretically and see if these results agree with each other

Method and Apparatus

Apparatus

- A frequency generator

- An oscilloscope

  - This is used to calculate Vin and VR

- A resistor

- A capacitor

- Wire to connect the resistor and capacitor, as well as connecting the oscilloscope and frequency generator to the circuit

- A digital multi-meter

  - The multi-meter is used to calculate the resistance of the resistor

A diagram of the apparatus used to collect data for Vin(V) and VR (V)

Diagram 1

A diagram of the multi-meter used to measure RΩ

Method

Firstly, 2 leads are used to connect the resistor to the multi-meter. This was done by plugging one lead in COM and the other lead into VΩ. Now the other ends of the leads connect to the resistor. The function selection switch was changed to the Ω function to turn the multi-meter into an ohmmeter. The measurement on the multi-meter was recorded after using the range button to get a better approximation for the resistance. After this measurement was made, the circuit in Diagram 1 was constructed. The resistor was connected to the capacitor and the function generator connected to both in series. Both channels from the oscilloscope were used. Channel 1 was connected across the resistor and capacitor (to measure Vin) and Channel 2 across the resistor (to measure VR). Next, the Vpeak to peak setting on the oscilloscope was turned on by pressing the Measure button and selecting Ch1, then using the multipurpose button to select Peak-Peak. Then the function generator is connected in series to the resistor and capacitor. The peak-to-peak voltage on the frequency generator was set to 8 V. The frequency was then set to any frequency value from 2 to 8 kHz. That is, 2kHz ≤f≤8kHz

The oscilloscope then displays Vin as Vpeak to peak for Channel 1 and VR for Channel 2. These values were recorded along with the period on the oscilloscope (Channels 1 and 2 read the same period). The frequency was changed, and the measurements were taken once more for a total of 8 readings.

Data and Results

Value for R

The following value for R was obtained

R=218.000±2.180 Ω

Calculation:

The value for R was measured at R=218.0 Ω.

This was a digital reading, meaning this is a rectangular PDF, so

ureading=218.05-217.9523=0.02887

Along with this, the machine is rated at 1%, so

umachine=218.0×1%=2.180

Finally,

utotal=ureading2+umachine2=0.028872+2.1802=2.180

Yielding the result above

Tables

Table 1: The raw data collected: Vin (V), VR (V) and T(s) are from the oscilloscope and f (kHz) is from the frequency generator.

A table showing the values for Vin (V) and VR (V) corresponding to a measured T (s) from the oscilloscope and f (kHz) from the frequency generator

| f (kHz) | Tos×10-4 | Vin (V) | VR (V) |
| --- | --- | --- | --- |
| 2.372 | 4.220 | 8.08 | 0.630 |
| 5.616 | 1.778 | 8.08 | 1.45 |
| 7.951 | 1.252 | 8.08 | 2.04 |
| 4.619 | 2.164 | 8.08 | 1.28 |
| 6.560 | 1.526 | 8.08 | 1.76 |
| 3.121 | 3.204 | 8.08 | 0.900 |
| 7.439 | 1.346 | 8.08 | 1.96 |
| 5.069 | 1.970 | 8.08 | 1.40 |

We convert the frequency from the frequency generator to a period, by T=1f

leading to:

Table 2: The values for 1f=T (s) for the frequency generator

A table showing the frequency (kHz) as a period (s)

| f (kHz) | Ts×10-4 |
| --- | --- |
| 2.372 | 4.216 |
| 5.616 | 1.780 |
| 7.951 | 1.258 |
| 4.619 | 2.165 |
| 6.560 | 1.524 |
| 3.121 | 3.204 |
| 7.439 | 1.344 |
| 5.069 | 1.973 |

Example calculation:

For f=2.372 kHz, this is 2.372×103=2372 Hz.

Then T=1f=12372=0.0004216=4.216×10-4s

This leads to the following ranges for Tf and To

Table 3: The intervals for the period from the frequency generator (Tf) (s) compared to the period (s) shown on the oscilloscope (To)

The values for the period (s) from the frequency generator and the period of the oscilloscope (s), with uncertainty applied to the best estimate

| Tfs×10-4 | Tos×10-4 |
| --- | --- |
| 4.174≤Tf≤4.258 | 4.136≤To≤4.304 |
| 1.762≤Tf≤1.798 | 1.742≤To≤1.814 |
| 1.245≤Tf≤1.271 | 1.227≤To≤1.277 |
| 2.143≤Tf≤2.187 | 2.121≤To≤2.207 |
| 1.509≤Tf≤1.539 | 1.496≤To≤1.557 |
| 3.172≤Tf≤3.236 | 3.140≤To≤3.268 |
| 1.331≤Tf≤1.357 | 1.319≤To≤1.373 |
| 1.953≤Tf≤1.993 | 1.931≤To≤2.009 |

Example Calculation:

There are two parts

- The uncertainty of Tf

For Tf=4.216×10-4, the frequency was f=2.372kHz (as by Table 2)

So f=2372Hz. The frequency generator was rated at 1%, so

umachine=2372×1%=23.72Hz

This was a digital reading, so this is a rectangular PDF so

ureading=2372.5-2371.523=0.2887Hz

Thus utotal=23.722+0.28872=23.72Hz

Now T=1f⇒T=f-1⇒uT=T-uff2=Tuff

=4.216×10-423.722372=4.216×10-6s

So 4.174×10-4≤Tf≤4.258×10-4

- The uncertainty of To

For To=4.220×10-4:

The oscilloscope was used for this and has a rating of 2%, thus

umachine=4.220×10-4×2%=8.440×10-6s

This was a digital reading, so this is a rectangular PDF, so

ureading=4.2205×10-4-4.2195×10-423=2.886×10-8s

So utotal=8.440×10-62+2.886×10-82=8.440×10-6s

Thus 4.136×10-4≤To≤4.304×10-4

Now we calculate VinVR2and 1f (displayed by the oscilloscope)

Table 3: The values for VinVR2,and (1f)^2=(To)^2 (s) calculated using the values from Table 1. We want these values because VinVR2=12πRC21f2+1 models a straight line with m = 12πRC2

and Y=VinVR2and x=1f2

A table showing the values for VinVR2 and To2(s2)

| To (s) ×10-4 | Vin (V) | VR (V) | VinVR2 | To2 (s2)×10-8 |
| --- | --- | --- | --- | --- |
| 4.220 | 8.08 | 0.630 | 164 | 17.8 |
| 1.778 | 8.08 | 1.45 | 31.1 | 3.16 |
| 1.252 | 8.08 | 2.04 | 15.7 | 1.57 |
| 2.164 | 8.08 | 1.28 | 39.8 | 4.68 |
| 1.526 | 8.08 | 1.76 | 21.1 | 2.33 |
| 3.204 | 8.08 | 0.900 | 80.6 | 10.3 |
| 1.346 | 8.08 | 1.96 | 17.0 | 1.81 |
| 1.970 | 8.08 | 1.40 | 33.3 | 3.88 |

Example Calculation:

For Vin=8.08 and VR=0.630

VinVR2=8.080.6302≈164

For T=4.216×10-4 kHz = 0.4216 Hz

So T2=0.42162=0.178=17.8×10-2 Hz2

Graphs

Graph 1: VinVR2 plotted against To2s2 from Table 3

VinVR2 for the period provided by the generator

VinVR2 plotted against To2s2 (the period displayed on the oscilloscope)

To2s2 displayed on the oscilloscope

Analysis and Discussion

Calculating C using the gradient for Graph 1

Using the least squares method for Graph 1 to calculate m:

m=89.100±3.615×107

Now m=12πRC2, so:

89.100×107=12π218.000C2⇒189.100×107=436πC⇒C=2.445×10-8F

To calculate the uncertainty of C, first note that:

m12=12πRC⇒m-12=2πRC⇒12πm-12R-1=C

So

uC=C{-12umm}2+{-1uRR}2

=2.44×10-7-3.615×1072×89.100×107

2+-2.180218.02=5.58×10-9F

Concluding that C=2.4400±0.0558×10-7F

m = 891006515.5891 +/- 36146151.7437		10^8 10^7

c = -0.4969 +/- 2.8029

Checking the Validity of the Result using The Intercept

Using the least squares method for Graph 1 to calculate c:

c=-0.6376 ±2.8497

Mathematically, we need c=0, and we have -3.4873 ≤c ≤2.2121 so this is a valid result

Calculating the Theoretical Value for C

Checking the Validity of the Period displayed by the Oscilloscope

We calculate the uncertainty for the frequency for both the oscilloscope and frequency generator

Example calculation:

Oscilloscope: The oscilloscope is rated at 2%. Thus for

Conclusion and Recommendations

Conclusion

Discussion

The calculation using the gradient yielded: g=8,390±0,293m⋅s-2 so

8,097 ≤g ≤8,683

whereas with the y-intercept, g=13,60±1,58m⋅s-2 so

12,02 ≤g ≤15,18

so, these values do not lie within each other. Reasons for this could be the imprecise measurement of T, using a stopwatch that only shows a value to 2 decimal places and relying on the user’s judgement as to when one period is completed

Sources of Uncertainty

- Judgement as to when one period is completed

- Uncertainty associated with the internal calibration of the stopwatch

- Uncertainty associated with the stop watch’s figure

- Judgement of the distance between the centre of mass and the hole (r)

Ways to Improve the Experiment

- Use a more precise timer

- Have some string attached to the timer and the metre stick so as to begin the timer just as the stick is released or some software that tracks when the metre stick passes its initial condition